Since the lander will be controlled manually from earth by a joystick, a panoramic periscope camera will be needed to see how to guide the lunar/rover. A Surface Stereo Imager or SSI will serve as the Landers "eyes" for the lunar mission, providing high-resolution, stereo, panoramic images of the lunar surface. Using an advanced optical system, SSI will survey the lunar surface for geological context, provide range maps in support of sampling operations, and make lunar dust cloud measurements. Resolution for the SSI will be twenty meters at best.   The SSI will be mounted on top of an extended mast, which is attached to the lander/rover. SSI simulates the human eye with a two optical lens system that will give a three-dimensional view of the lunar surface. This instrument will also simulate the resolution of human eyesight using a charged-coupled device that produces high density 1024 x 1024 pixel images. The SSI actually exceeds the capabilities of the human eye by using optical and infrared filters, allowing multi-spectral imaging at twelve wavelengths of geological and atmospheric interest. Using narrow-band imaging of the Sun, the imager will estimate density of the lunar dust. SSI will also look at the Lander itself to assess any damage to the spacecraft and also the amount of wind-blown dust deposits. This is critical to engineers who are concerned about any vehicle damage, the amount of deposited dust on the solar panels, and also associated power degradation.

Shown below is a picture of an SSI that will be used on the Lander/rover.  The two black squares on the front are the cameras “eyes”.

Shown below is a picture of Mars taken by the SSI’s predecessor the Imager for Mars Pathfinder or IMP. The images taken by the SSI will be even clearer than this.

For this project, it is assumed that the lander/rover will be set down at a given, known location, Shackleton’s Crater. Knowing this, the lander/rover will be navigated by using the SSI to look around at landmarks. These landmarks will be compared to detailed maps of the moon to determine the lander/rovers exact location. Shown below is a picture of Shackleton’s Crater.