For landing the Cyclops, a Laser Imaging Detection And Ranging (LiDAR) system will be used. LiDAR has the capabilities of measuring distance, speed, rotation, and the chemical composition and concentration of the lunar surface. During flight, the LiDAR sensor pulses a narrow, high frequency laser beam toward the moon. The LiDAR sensor then records the time difference between the emission of the laser beam and the return of the reflected laser signal to the spacecraft. The reflected laser light from the ground follows the reverse optical path and is directed into a small Cassegrainian telescope. A moving mirror produces a conical sampling pattern beneath the spacecraft over a thirty degree wide swath, thus permitting the collection of topographic information over a strip approximately three hundred meters (approximately 1000 feet) in width from the nominal six hundred meter (2000 feet) data collection altitude. The LiDAR has two sounding modes: active and acoustic. In order to locate and characterize ice and dust hazes, light pulses are emitted and their return is timed. This is done to a level of 2 to 3 kilometers. To record the returning sound, an acoustic device known as a Mars Microphone will be used.

The LiDAR was designed in a bi-static scheme with a separate transmitter and two receiver channels. Its three assemblies consist of a sensor assembly, an electronic assembly, and the interconnecting cables. The total mass of the instrument is nine hundred and forty grams (approximately 2 pounds) and the total volume is nine hundred and fifty cubic centimeters in both assemblies. Power consumption is a maximum of four Watts during ten minute measurements. Engineers designed the LiDAR to operate in the temperature range from -100 to 50 degrees Celsius (-148 to 122 degrees Fahrenheit) under lower pressures, near one millibar. Since the LiDAR is only used for landing the Cyclops, the colder lunar temperatures will not effect operations of the LiDAR.

The laser transmitter is a gallium-aluminum-arsenic pulse laser diode emitting one micro Joule in twenty nanosecond long pulses at a wavelength of eight hundred and ninety nanometers. The laser beam has a linear polarization and a two milli-radian divergence with an output clear aperture of 54x34 mm2. The laser operates with a repetition rate of approximately 20 to 25 kHz.

Outside daylight background irradiance is the main source of noise inside the one mrad field-of-view of the receiver channel. Background irradiance is reduced by an optical interference filter with a bandwidth of ten nm (FWHM) at eight hundred and ninety nm with fifty percent transmission and a clear aperture of fifteen millimeter.

The full sounding distance of the LiDAR is seven hundred and fifty meters (2,461 feet) with five meters resolution along the trace or the thirty two second total integrating time. The single cycle has a ten minute duration and consists of three active measurements by each channel simultaneously and twelve passive measurements of sky brightness. A picture of the LiDAR system is shown in figure x of the appendix.

To capture a view of the penetrator dispersion and of the Cyclops landing, a Mars Descent Imager will be mounted on the bottom of Cyclops. This imager is the same imager that was used on the 1998 Mars surveyor lander. The MARDI can provide imager over three orders of magnitude in scale, correlating post-landing surface images to pre-landing orbital images. The MARDI uses short exposure time to reduce image blurring from spacecraft motion. It has a high-definition, video like data acquisition, with 1600x1200 pixels at five frames/sec, and a large internal storage of 256 MByte SRAM and an eight GByte flash. Since the imager will only be used during the landing, the MARDI will not need any thermal provisions. A picture of the MARDI is shown in figure x of the appendix.

Since the Cyclops will be controlled manually from earth by a joystick, a panoramic periscope camera will be needed to see how to guide the lunar/rover. A Surface Stereo Imager or SSI will serve as the Landers "eyes" for the lunar mission, providing high-resolution, stereo, panoramic images of the lunar surface. Using an advanced optical system, SSI will survey the lunar surface for geological context, provide range maps in support of sampling operations, and make lunar dust cloud measurements. Resolution for the SSI will be twenty meters at best.   The SSI will be mounted on top of an extended mast, which is attached to the Cyclops. SSI simulates the human eye with a two optical lens system that will give a three-dimensional view of the lunar surface. This instrument will also simulate the resolution of human eyesight using a charged-coupled device that produces high density 1024 x 1024 pixel images. The SSI actually exceeds the capabilities of the human eye by using optical and infrared filters, allowing multi-spectral imaging at twelve wavelengths of geological interest. Using narrow-band imaging of the Sun, the imager will estimate density of the lunar dust. SSI will also look at the Lander itself to assess any damage to the spacecraft and also the amount of wind-blown dust deposits. This is critical to engineers who are concerned about any vehicle damage, the amount of deposited dust on the vehicle, and also associated power degradation.

Shown in figure x in the appendix is a picture of an SSI that will be used on the Cyclops.  The two black squares on the front are the cameras “eyes”. Shown in figure 3 in the appendix is a picture of Mars taken by the SSI’s predecessor the Imager for Mars Pathfinder or IMP. The images taken by the SSI will be even clearer than this.

The SSI will be mounted on a one meter high mast which is raised by an electric motor. There will be a light mounted on the front of the Cyclops for operations in the darker areas.

For this project, it is assumed that the Cyclops will be set down at a given, known location, around Shackleton Crater. Knowing this, the Cyclops will be navigated by using the SSI to look around at landmarks. These landmarks will be compared to detailed images of the moon to determine the Cyclops exact location. Shown in figure 4 in the appendix is a picture of Shackleton Crater.

The on-board control system will be built around a BAE RAD750 processor, which is manufactured by BAE systems. This processor was selected because it is made for use in high radiation environments such as the moon. The Mars Reconnaissance Orbiter currently has a BAE RAD750 on board. According to development information, this processor will suffice the Cyclops needs so long as the Cyclops is operated at slow speeds. A picture of the RAD 750 is shown in figure X of the appendix.

To drive the Cyclops, six heated electric motors will be used. For steering, four heated electric steering motors will be used. To save energy, Cyclops will not steer and drive at one time. Instead, it will completely stop, steer then begin to move again.  Since Cyclops will be operated manually from the earth, the current operator, using the SSI, will serve as the hazard detection and avoidance system. Should the Cyclops lose communications with ground control, the system will place itself into a hold mode until communications with earth ground control can be reestablished.

After calculating the mass flow rate and the side velocity from the ACS thrusters, the horizontal distance that Cyclops will be able to travel with the amount of propellant supplied is approximately 1.7km. Therefore, Cyclops will descend to approximately 1.6km from the edge of Shackleton at an altitude of five km, disperse fifteen penetrators, then thrust 1.6km to the outside of Shackleton and land on its wheels. Calculations for this maneuver are given in the appendix.

References:

LIDAR:

http://mars.jpl.nasa.gov/msp98/lander/lidar.html

http://www.iki.rssi.ru/lidar/english/instr.htm

MARDI:

http://www.msss.com/mars/surveyor/mardi.html

http://www.msss.com/mars/surveyor/ms98/lander/mardi/abstract/abstract.html

Mars Rover Info:

http://mars.jpl.nasa.gov/MPF/mpf/sci_desc.html#IMP

http://mpfwww.jpl.nasa.gov/msp98/mvacs/live/

http://cmex.ihmc.us/SiteCat/sitecat2/pathfind.htm

Processor Info:

http://en.wikipedia.org/wiki/RAD750

Surface Stereo Imager (SSI):

http://mars.jpl.nasa.gov/msp98/lander/lidar.html

http://www.nasa.gov/mission_pages/phoenix/spacecraft/ssi.html

http://mpfwww.jpl.nasa.gov/MPF/imp/anatomy/filters.html