For landing the lander, a LIght Detection And Ranging (LIDAR) system will be used. LIDAR has the capabilities of measuring distance, speed, rotation, and the chemical composition and concentration of the lunar surface. During the flight, the LIDAR sensor pulses a narrow, high frequency laser beam toward the moon. The LIDAR sensor then records the time difference between the emission of the laser beam and the return of the reflected laser signal to the spacecraft. The reflected laser light from the ground follows the reverse optical path and is directed into a small Cassegrainian telescope. A moving mirror produces a conical sampling pattern beneath the spacecraft over a thirty degree wide swath, thus permitting the collection of topographic information over a strip approximately three hundred meters (approximately 1000 feet) in width from the nominal six hundred meter (2000 feet) data collection altitude. The LIDAR has two sounding modes: active and acoustic. In order to locate and characterize ice and dust hazes, light pulses are emitted and their return is timed. This is done to a level of 2 to 3 kilometers. To record the returning sound, an acoustic device known as a Mars Microphone will be used. This information will be fed into to some computer controls system which will then be used to control the “landing thrusters”.

The LIDAR was designed in a bistatic scheme with a separate transmitter and two receiver channels. Its three assemblies consist of a sensor assembly, an electronic assembly, and the interconnecting cables. The total mass of the instrument is nine hundred and forty grams (approximately 2 pounds) and the total volume is nine hundred and fifty cubic centimeters in both assemblies. Power consumption is a maximum of four Watts during ten minute measurements. Engineers designed the LIDAR to operate in the temperature range from -100 to 50 degrees Celsius (-148 to 122 degrees Fahrenheit) under lower pressures, near one millibar. It is connected to the board by an RS-422 interface and generates science data up to 25.6 Kb per day for a standard operation scenario.

The LIDAR also has a high pressure variometer for studying the correlation of optical atmospheric characteristics with very low frequency sounds (infrasounds) and changing pressures.

The laser transmitter is a galium-aluminum-arsenic pulse laser diode emitting one microJoule in twenty nanosecond long pulses at a wavelength of eight hundred and ninety nanometers. The laser beam has a linear polarization and a two milliradian divergence with an output clear aperture of 54x34 mm2. The laser operates with a repetition rate of approximately 20 to 25 kHz.

Outside daylight background irradiance is the main source of noise inside the one mrad field-of-view of the receiver channel. Background irradiance is reduced by an optical interference filter with a bandwidth of ten nm (FWHM) at eight hundred and ninety nm with fifty percent transmission and a clear aperture of fifteen millimeter.

The full sounding distance of the LIDAE is seven hundred and fifty meters (2,461 feet) with five meters resolution along the trace or the thirty two second total integrating time. The single cycle has a ten minute duration and consists of three active measurements by each channel simultaneously and twelve passive measurements of sky brightness.

A picture of the LIDAR system is shown below: