Concept Description Document Approval:

The undersigned agree that the attached Concept Description Document as marked will be the basis the UAH IPT 2008 Design Competition.  From this time forward, any questions or clarifications concerning the concept description document to the Customer shall be submitted in writing and the answer distributed to all UAH IPT’s in writing.

To change the Concept Description Document Prior to May 2, 2008 shall require that the change be stated in writing and that a person authorized by every one of the signers below endorse the change with their signature.  The revision will be labeled uniquely and distributed to all teams simultaneously.

The original of this document will be kept on file with the UAH Project Director.  All signers will receive a copy of the original document.

__________________/__________

Nick Case, UAH IPT Team D, Project Office

__________________/__________

Eddie Kiessling, UAH IPT Team E, Project Office

__________________/__________

Matt Isbell, UAH IPT Team E, Project Office

__________________/__________

David Jacobson, UAH IPT Project Customer

__________________/__________

Michael P.J. Benfield, UAH IPT Project Leader (Class Instructor)

__________________/__________

Robert A Frederick, UAH IPT Project Director

- Scope.  This document outlines the specifications for the Lunar Exploration Transportation System (CYCLOPS).  The CYCLOPS project is to develop a lunar lander mission for NASA that provides the flexibility to conduct different scientific investigations and technology validation tasks at different areas on the moon’s surface.  For this study only one lander is envisioned.  The mobility of the lander system is subject to trade.  For the purposes of this project, the mission begins with terminal descent.  The project goal is to design the transportation system to accomplish the scientific and technology objectives, not the science and technology itself.

- Customer Requirements.  The following requirements were given by the customer as Level 1 Requirements.

  - The CYCLOPS shall use an Atlas V-401 EPF shroud configuration with a total landed mass of 997.4 kg.

    - The CYCLOPS shall be designed so that 64.6 kg of the total landed mass is devoted to the propulsion system dry mass.  Table 1 details this mass allocation to the propulsion subsystems.

    - The CYCLOPS propulsion system shall be designed to accommodate 159.5 kg of hydrazine (N2H4) propellant and 2.0 kg of helium.  The propellant shall be housed in two spherical propellant tanks, each 0.55 m in diameter.  The helium shall be housed in 2 spherical tanks, each 0.4 m in diameter.

    - The CYCLOPS propulsion system shall be designed for two (2) MR-80B monopropellant liquid rocket engines.

    - The CYCLOPS attitude control system shall be designed for twelve (12) MR-106 monopropellant thrusters.

  - The CYCLOPS shall be designed for its first mission to be at a polar location.

  - The CYCLOPS shall be designed with the capability to land at other lunar locations.

  - The CYCLOPS shall minimize cost across the design.

  - The CYCLOPS shall launch to the moon NLT September 30, 2012.

  - The CYCLOPS shall have the capability to move on the surface.

  - The CYCLOPS shall be designed to survive for one year on the surface of the moon.

  - The CYCLOPS shall survive the proposed concept of operations.

  - The CYCLOPS shall be capable of meeting both the Science Mission Directorate (SMD) and the Exploration Systems Mission Directorate (ESMD) objectives.

  - The CYCLOPS shall land to a precision of ± 100m 3 sigma of the predicted location.

    - The CYCLOPS shall provided guidance, navigation, and control for the terminal descent phase, beginning at 5 km above the surface of the moon.

    - The Cyclops shall utilize a LiDAR system for landing the Cyclops

    - The Cyclops shall utilize the Mars Descent Imager for capturing views of the penetrator dispersion and of the landing site.

    - The Cyclops shall utilize a Surface Stereo Imager for manual guidance of the Cyclops and for analyzing lunar dust.

    - The Cyclops shall utilize an BAE RAD750 processor for all controls processing.

  - The CYCLOPS shall be capable of landing at a slope of 12 degrees (slope between highest elevated leg of landing gear and lowest elevated leg).

  - The CYCLOPS shall be designed for g-loads during lunar landing not to exceed the worst case design loads for any other phase of the mission (launch to terminal descent).

    - The CYCLOPS shall be designed to withstand g-loads with respect to stiffness only.

    - The CYCLOPS design shall not be concerned with frequency responses or frequency loads.

- Concept Design Constraints.  The following constraints are placed on the CYCLOPS design.

  - The CYCLOPS shall be designed to interface with the Atlas V-401 EPF shroud configuration launch vehicle per the Atlas Launch System Mission Planner's Guide, Rev 10a, January 2007, CLSB-0409-1109.

  - The CYCLOPS shall be designed to survive the lunar cruise environment for up to 28 days per NASA Technical Memorandum 82478, Space and Planetary Environment Criteria Guidelines for Use in Space Vehicle Development, Volume 1. Including but not limited to the following environment obstacles

    - Radiation

    - Thermal

    - Micrometeoroids

  - The CYCLOPS shall be designed to survive the lunar surface environment at both the polar and equatorial regions.

    - This includes temperatures ranging from 107 Celsius to -223  Celsius

    - Including conditions stated in section 3.2

  - The CYCLOPS shall maximize the use of off-the-shelf technology.  Off-the-shelf technology shall have a technology readiness level of 9.

    - Readiness of level 9 states it is an actual “Flight Proven” system through successful mission operations.

  - The CYCLOPS shall be designed to accomplish the maximum surface objectives outlined below.

- Figures of Merit.  The following Figures of Merit (FOM) will be used to evaluate the CYCLOPS design concepts.

  - Number of surface objectives accomplished (as outlined below)

  - Percentage of mass allocated to payload

  - Ratio of objectives (SMD to ESMD) validation (2 to 1 being the goal)

  - Efficiency of getting data in stakeholders hands vs. capability of mission

  - Percentage of mass allocated to power system

  - Ratio of off-the-shelf hardware to new development hardware

- Surface Objectives.  The following surface objectives were provided by the customer.

  - Single site goals - Geologic context

    - Determine lighting conditions every 2 hours over the course of one year

    - Determine micrometeorite flux

    - Assess electrostatic dust levitation and its correlation with lighting conditions

  - Mobility goals

    - Independent measurement of 15 samples in permanent dark and 5 samples in lighted terrain

    - Each sampling site must be separated by at least 500 m from every other site

    - Minimum: determine the composition, geotechnical properties and volatile content of the regolith

    - Value added: collect geologic context information for all or selected sites

    - Value added: determine the vertical variation in volatile content at one or more sites

    - Assume each sample site takes 1 earth day to acquire minimal data and generates 300 MB of data

  - Instrument package baselines

    - Minimal volatile composition and geotechnical properties package, suitable for a penetrometer, surface-only, or down-bore package: 3 kg

    - Enhanced volatile species and elemental composition (e.g. GC-MS): add 5 kg

    - Enhanced geologic characterization (multispectral imager + remote sensing instrument such as Mini-TES or Raman): add 5 kg

Table 1.  Propulsion Dry Subsystem Mass

| Subsystem | Number | Mass/Unit (kg) | Total Mass (kg) |
| --- | --- | --- | --- |
| Propellant Tank | 2 | 5.1 | 10.2 |
| Helium Tank | 2 | 5.2 | 10.4 |
| Helium | 2 | 1.0 | 2.0 |
| Main Engine | 2 | 7.9 | 15.8 |
| ACS Thruster | 12 | 0.5 | 6.0 |
| Components | 1 | 14.1 | 14.1 |
| Contingency | 1 | 6.1 | 6.1 |