Concept Description Document Approval:

The undersigned agree that the attached Concept Description Document as marked will be the basis the UAH IPT 2008 Design Competition.  From this time forward, any questions or clarifications concerning the concept description document to the Customer shall be submitted in writing and the answer distributed to all UAH IPT’s in writing.

To change the Concept Description Document Prior to May 2, 2008 shall require that the change be stated in writing and that a person authorized by every one of the signers below endorse the change with their signature.  The revision will be labeled uniquely and distributed to all teams simultaneously.

The original of this document will be kept on file with the UAH Project Director.  All signers will receive a copy of the original document.

__________________/__________

Group Member NAME, UAH IPT Team D, Project Office

__________________/__________

Group Member NAME, UAH IPT Team E, Project Office

__________________/__________

Group Member NAME, UAH IPT Team E, Project Office

__________________/__________

David Jacobson, UAH IPT Project Customer

__________________/__________

Michael P.J. Benfield, UAH IPT Project Leader (Class Instructor)

__________________/__________

Robert A Frederick, UAH IPT Project Director

- Scope.  This document outlines the specifications for the Lunar Exploration Transportation System (LETS).  The LETS project is to develop a lunar lander mission for NASA that provides the flexibility to conduct different scientific investigations and technology validation tasks at different areas on the moon’s surface.  The project goal is to design the transportation system to accomplish the scientific and technology objectives, not the science and technology itself.

- Customer Requirements.  The following requirements were given by the customer as Level 1 Requirements.

  - The LETS shall have a landed mass of 1450 kg ± 100 kg.

  - The LETS shall be design for its first mission to be at a polar location

  - The LETS shall be designed with the capability to land at other lunar locations.

  - The LETS shall minimize cost across the design.

  - The LETS shall launch to the moon no later than September 30, 2012.

  - The LETS shall have the capability to move on the surface.

  - The LETS shall be designed to survive for one year on the surface of the moon.

  - The LETS shall survive the proposed concept of operations.

  - The LETS shall be capable of meeting both the Science Mission Directorate (SMD) and the Exploration Systems Mission Directorate (ESMD) objectives.

  - The LETS shall land to a precision of ± 100m 3 sigma of the predicted location.

  - The LETS shall be capable of landing at a slope of 12 degrees (slope between highest elevated leg of landing gear and lowest elevated leg).

  - The LETS shall be designed for g-loads during lunar landing not to exceed the worst case design loads for any other phase of the mission (launch to terminal descent).

- Concept Design Constraints.  The following constraints are placed on the LETS design.

  - The LETS shall be designed to interface with the Atlas V-401 Launch Vehicle per the Atlas Launch System Mission Planner's Guide, Rev 10a, January 2007, CLSB-0409-1109.  The shroud configuration for the LETS shall be determined by each individual team, giving three standard options provided for the 400 series.

  - The LETS shall be designed to survive the lunar cruise environment for up to 28 days per JPL Standard. Including but not limited to the following environment obstacles

- Radiation

- Thermal

- Micrometeoroids

  - The LETS shall be designed to survive the lunar surface environment at both the polar and equatorial regions.

    - This includes temperatures ranging from 107 ͦC to -153  ͦC

    - Including conditions stated in section 2.11

  - The LETS shall maximize the use of off-the-shelf technology.  Off-the-shelf technology shall have a technology readiness level of 9.

    - Readiness of level 9 states it is an actual “Flight Proven” system through successful mission operations.

  - The LETS shall be designed to operate for one year.

  - The LETS shall be designed to accomplish the maximum surface objectives outlined below.

- Figures of Merit.  The following Figures of Merit (FOM) will be used to evaluate the LETS design concepts.

  - Number of surface objectives accomplished (as outlined below)

  - Percentage of mass allocated to payload

  - Ratio of objectives (SMD to ESMD) validation

  - Efficiency of getting data in stakeholders hands vs. capability of mission

  - Percentage of mass allocated to power system

  - Ratio of off-the-shelf to new Development

- Surface Objectives.  The following surface objectives were provided by the customer.

  - Single site goals - Geologic context

    - Determine lighting conditions every 2 hours over the course of one year

    - Determine micrometeorite flux

    - Assess electrostatic dust levitation and its correlation with lighting conditions

  - Mobility goals

    - Independent measurement of 15 samples in permanent dark and 5 samples in lighted terrain

    - Each sampling site must be separated by at least 500 m from every other site

    - Minimum: determine the composition, geotechnical properties and volatile content of the regolith

    - Value added: collect geologic context information for all or selected sites

    - Value added: determine the vertical variation in volatile content at one or more sites

    - Assume each sample site takes 1 earth day to acquire minimal data and generates 300 MB of data

  - Instrument package baselines

    - Minimal volatile composition and geotechnical properties package, suitable for a penetrometer, surface-only, or down-bore package: 3 kg

    - Enhanced volatile species and elemental composition (e.g. GC-MS): add 5 kg

    - Enhanced geologic characterization (multispectral imager + remote sensing instrument such as Mini-TES or Raman): add 5 kg