Atlas Launch System Mission Planner's Guide

Revision 10a, January 2007

CLSB-0409-1109

The    static    payload    envelope    defines    the    usable    volume    for    a    spacecraft.    This    envelope    represents    the

maximum    allowable    spacecraft    static    dimensions    (including    manufacturing    tolerances)    relative    to    the

spacecraft    and    payload    adapter    interface.    For    clearances    between    the    spacecraft    and    PLF,    primary

clearance concerns are for dynamic deflections of the spacecraft and PLF and the resulting relative loss of

clearance between these components. For clearances between the spacecraft and PLA, primary envelope

concerns    are    for    access    to    the    mating    components    and    payload    separation    system    for    integration    and

installation operations, motion of the payload separation system during its operation, and movement of the

spacecraft and launch vehicle after separation of the spacecraft. For all areas of the static payload envelope,

clearance layouts and analyses are performed for each spacecraft configuration, and if necessary, critical

clearance locations are measured during spacecraft and launch vehicle integration operations to ensure a

positive    clearance    is    maintained.    Overall    views    of    static    payload    envelopes    for    Atlas    V    400    series    launch

vehicle configurations are shown in Figure 4.1.1-3 and for Atlas V 500 series launch vehicle configurations

are shown in Figure 4.1.1-4. Detailed descriptions are included in Appendix D.

Figure 4.1.1-3:          Atlas V 4-m Static Payload Envelope

Dimensions: mm [in]

4-m LPF

4-m EPF

4-m XEPF

AMPG10a_F040101_03a

Cleared for Public Release Through OFOISR

Page 4-4