6U CompactPCI single-board computer

6U availability: Available now

Supports variable-memory configurations

SRAM: 		16 MBytes standard with options to 48 Mbytes and ECC

Device sparing

SUROM: 	64 KBytes + ECC

EEPROM for engineering (256 KBytes)

PROM for flight units (64 KBytes)

EEPROM: 	4 MBytes + ECC

6U specifications

Form factor

CompactPCI 6U (233 mm x 160 mm)

Weight: 1000 to 1220 grams, varies with memory

Memory

16-48 MBytes RHSRAM,4 MBytes EEPROM

Radiation-hardness

Total dose: >100 Krad (Si)

SEU: 1.9 E-4 errors/card-day (90% W. C. GEO)

varies with orbit

Latchup: Immune

Performance

>260 Dhrystone 2.1 MIPS @ 132 MHz

4.3 SPECint95 4.6 SPECfp95 at 132 MHz

Power supply

5V ± 10% and 3.3V ± 5%

(2.5V generated via onboard regulator)

Power dissipation

11-14W

Rail temp range

-28°C to +70°C