## Payload

To achieve the scientific goals of our mission our team has decided to use Lunar Penetrators Exploration System (for our multiple site goals) and science instrument packages on Daedalus.  Daedalus will have three science packages for a single site, utilized over a period of 1 year.  A minimal volatile composition and geotechnical properties package with a mass of 3 kg, an enhanced volatile species and elemental composition with a mass of 5 kg, and an enhanced geologic characterization with a mass of 5 kg.  These packages supported by a drill mechanism, arm and scoop, and witness plates will successfully determine:

- Lighting conditions every 2 hours over the course of one year

- Micrometeorite flux

- Electrostatic dust levitation and its correlation with lighting conditions

Below is the list of payload instrumentation including with mass and power requirements.

Table 1

| Daedalus Payload | Mass kg | Power W |
| --- | --- | --- |
| Stereo imaging system or Radar or Lidar | 0.8 | 6 |
| Mast for stereo imaging system | 3.5 | 9.5 |
| Drill and drill deployment mechanism | 20 | 30 |
| Arm | 13 | 43 |
| Scoop | 0.5 | 0 |
| Mass Spectrometer | 19 | 75 |
| Geotechnical Experiments - cone penetrometer | 1.5 | 0 |
| Geotech - bearing plate | 1 | 0 |
| Geotech - shear vane | 0.5 | 0 |
| Magnets | 0.5 | 0 |
| XRD/XRF | 2 | 10 |
| Total | 62.3 | 173.5 |
| Estimated |  | 168 |

The penetrators will be dispersed in the Shackleton Crater amid the mostly dark regolith surface.  By using tested instrumentation from the Deep Space 2 penetrators (TRL 6) and LUNAR-A penetrators (Space Qualified), LunaTech was able to parallel new projects such as Lunar EX and MoonLite.  The design concept incorporates LUNAR-A’s architecture of a ballistic missile shape, but cutting off the propulsion system.

Table 2

| Penetrator Requirements |  |
| --- | --- |
| Impact Deceleration | Up to 10000g |
| Impact Angle (between impact velocity vector and tangent to surface) | ~90 degrees |
| Attack Angle (between penetrator long axis and impact velocity vector) | ~<8 degrees |
| Penetration Depth into Regolith | 1-2m |
| Ambient Penetrator operating temperature | 50K to 100K |
| Mean Penetrator power (subsystems and payload) | 60mW |
| Mission Duration | 1.2 year |

In this way mass is conserved to allow for 22 penetrators with a mass of 14kg each.  They will have a diameter of 14cm and a length of 75cm.  Without an atmosphere to perform

Aerobraking (DS-2), the lunar probes will have to be ejected from the carrier spacecraft with very little lateral velocity, at the appropriate height so that free fall will give the penetrators the proper impact velocity.  The penetrators will be spring loaded into chambers located on the bottom of Daedalus.  At 5km above the lunar surface 16 penetrators will be released.

There are geometrically sized rings that the chambers will be aligned.  The inner and outer most rings will launch the first deployment of penetrators.  The inner ring of chambers are angled at 2.37 degrees and the outer chambers at 10.41 degrees.  This will insure that all 16 penetrators will be a minimum of 500m from each other and allowing the smallest angle of attack.  The middle ring penetrators (6 total) will be launched 3km above the lunar surface in the sunlight, outside the Shackleton Crater.  To achieve a distance of 500m distance apart their attack angle will be 9.46 degrees.  For both deployments a same impact velocity is needed to reach our goal of the penetrators to burrow into the lunar surface 1-2m.  Different spring sized springs will be used for the two sets of deployment due to the difference in initial velocities.  Steel springs with spring constants of ???? will provide an initial velocity leaving Daedalous of  ???? for the first set, and spring constants of ???? to achieve ????  initial velocity.  With these initial velocities and impact velocity of ~350m/s can be acquired.

Equations Used:

Penetrator Impact Velocity

Penetrator Depth

G-Force

At this speed the penetrators will have to withstand a G-force of ~4500.  A vast amount of resources has been devoted to testing survival conditions of the instrumentation at such speeds.  These types of “missiles” are common within the defense sector and a (limited) range of components are available off-the-shelf that will survive impacts of >50,000g.  Each penetrator will include micro-seismometers, a geochemistry package, a water/volatiles detector, a heat flow experiment, and accelerometers and tilt sensors. These packages support:

- Lunar Seismology

  - A network of micro-seismometers could detect the presence and measure the size of a lunar iron core; determine crustal and basal fill thickness; the deep structure of lunar mantle; the origin and location of the enigmatic shallow moonquakes. These would benefit understanding of Moon’s residual magnetism; origin of Earth-Moon system; and evolution of planetary magnetic fields.

- Lunar Thermal Gradients

  - Passive thermometers would be capable of determining heat flow from the Lunar interior and information on inhomogeneity of crustal heat producing elements (U,K,Th).  This is relevant to understanding the Moon’s early history.  Lunar Water and other Volatiles Sensing Water sensors capable of detecting the presence, extent, and concentration of water and other volatiles (organic), would provide information on Lunar evolution and future Lunar resources. If possible, an isotope discrimination capability could also provide information on the origin of the water. Such information could also be relevant to astrobiology including the highly controversial panspermic theory of the origin of life on Earth.

- Geochemical Analysis

  - Provide ground truth for remote sensing XFA and multi-spectral imaging.

- Far Side

  - As there has not yet been a far side landing, and there is evidence that the near and far sides of the Moon may be quite different internally, this could determine differences in regolith, lunar interior structure and composition.

The LPES will be designed to operate for 1-1.2 years below the lunar surface. This has very significant consequences for total energy requirement.  It is not proposed to have a detached aft body surface element (unlike DS-2) and therefore all power must be generated internally.  Moreover, the temperature on the lunar surface is estimated to be <50K which is dramatically different then Mars where the DS-2 is based for.  The instruments are powered by Li-SOCL2 (super lithium) batteries with an expected lifetime of one year. The batteries (with a power density ~430 Wh/kg) will power the electronics that will in turn provide heat to keep the instruments warm.  A 400 MHz UHF hybrid telemetry system will be used for communication between the penetrators and the orbiter. The penetrator antennae will be 1 to 3 meters below the surface, but lunar regolith is relatively transparent to radio waves. The data transfer rate from the penetrators to the orbiter will be 1 kbit/s and will occur one hour each day when the orbiter is in position overhead.  The LPES will be constructed out of Carbon fiber reinforced plastic, with an Ogive nose.  Shock protection will be provided by high rigid epoxy-resin and glass micro-spheres.

  - Payload Appendix