Power System

Phase 2 Requirements:

Power Storage “Batteries”

- The Daedelus shall use all secondary rechargeable batteries.

- The Daedelus shall use 9 Lithium-Ion batteries, Sony 1860HC, each with a specific energy of 114.627 W-hr/kg.

- The Daedelus batteries shall have a Depth of Discharge of 20% and an Efficiency of 90%.

- The Daedelus batteries shall have 6 cells per battery string.

- The Daedelus batteries shall produce 5.6 W*hours per cell.

- The Daedelus batteries shall have 16 strings.

- The Daedelus batteries shall have 96 cells per battery.

- The Daedelus batteries shall have an individual length of 9.764-inches.

- The Daedelus batteries shall have an individual width of 7.362-inches.

- The Daedelus batteries shall have an individual height of 3.583-inches.

- The Daedelus batteries shall have a combined volume of 1.341 ft^3.

- The Daedelus batteries shall have an individual mass of 4.69 kg.

- The Daedelus batteries shall have a total mass of 42.21 kg.

- The Daedelus batteries shall have a cell capacity of 1.5 A*hours

- The Daedelus batteries shall have a total cell energy of 144 A*hours.

Power Generation “Solar Cells”

- The Daedelus shall use 3 solar cells derived from the Venus Express Spacecraft made of Gallium Arsenide. The solar cells shall be comprised of 2 identical sized solar panels and 1 smaller solar panel.

- The Daedelus solar cells shall have 2.75% degradation per year.

- The Daedelus solar cells 2 identical panels shall have a length of 1.778 m.

- The Daedelus solar cells 2 identical panels shall have a width of 0.8 m.

- The Daedelus solar cells 2 identical panels shall have a mass of 20.25 kg each.

- The Daedelus solar cells 2 identical panels shall produce a maximum of 410W each.

- The Daedelus solar cells 2 identical panels shall have a surface area of 2.822 m^2 each.

- The Daedelus solar cells 3rd and smallest panel shall have a length of .508 m.

- The Daedelus solar cells 3rd and smallest panel shall have a width of 0.229 m.

- The Daedelus solar cells 3rd and smallest panel shall have a mass of 5.786 kg.

- The Daedelus solar cells 3rd and smallest panel shall have a surface area of 0.4 m^2.

- The Daedelus solar cells 3rd and smallest panel shall produce a maximum of 117.143 Watts.

- The Daedelus solar cells shall produce a total 937.143W of electrical power.

- The Daedulus solar cells shall have a combined mass of 46.286kg.

- The Daedelus solar cells shall have a total surface area of 6.161 m^2.

Power Control “Power Control Unit/Power Regulation and Control”

- The Daedelus PCU shall have 2 Auxiliary Power Regulators per Solar Cell.

- The Daedelus PCU shall have 1 Battery Charge/Discharge Regulator per Battery.

- The Daedelus PCU shall use a STMicroelectronix ST0F269 digital signal processor and/or Microprocessor.

- The Daedelus PCU shall use Crydom CMX60D10 Solid State Relays.

- The Deadelus PCU shall use ON Semiconductor LM350 positive voltage regulators.

- The Daedelus PCU shall require 27.333 Amps and 1.207 Ohms of resistance.

- The Daedelus PCU shall have a total mass of 38.95kg.

- The Daedelus PCU shall have a total volume of 2.207 ft^3.

- The Daedelus wiring shall have a total mass of 9.5 kg.

System Duty Cycles

- The Daedelus shall be able to operate for 14 days of total darkness.

- The Daedelus shall make use of system duty cycles to limit the time and power consumed during the total darkness times.

- The Deadelus shall make us of 3 individual systems.

- The Daedelus first system shall operate for the entire dark period and provide constant priming power for the PCU.

- The Daedelus second power system will operate only 10% of the time, consisting of the Power System and the Thermal System.

- The Deadels third system shall operate only 1.25% of the time to take data and communicate with Earth.

- The Daedelus shall operate during the dark period for a total of 8530 W*hours.

Recommendations

- Design short circuit protection for PCU.

- Design the PCU with radiation protection for the Microprocessor.

- Design a smaller honeycombed area for each individual solar cell to allow for a denser cell packing, reducing the total surface area required.

- Determine the exact power consumed by the switching systems for a total loss of power.

References: Katherine has all proper MLA documentation for my sources.

- AIAA paper on Venus Express.

- MAE 468 book