- The LOW will require a stereo imaging system.

  - The LOW’s stereo imaging system will require a mass of 0.8 kg.

  - The LOW’s stereo imaging system will require power of 6.0 W.

  - The LOW’s stereo imaging system will include cameras.

  - The LOW’s stereo imaging system will be capable of taking 360° pictures.

- The LOW’s stereo imaging system will require a mast.

  - The LOW’s mast will require a mass of 3.5 kg.

  - The LOW’s mast will require power of 9.5 W.

  - The LOW’s mast will be deployed once at the initial site.

- The LOW’s stereo imaging system will require a illumination system.

  - The LOW’s illumination system will have four bulbs at 0.1 kg each.

  - The LOW’s illumination system will require a total mass of 0.4 kg.

  - The LOW’s illumination system will require power of 1.0 W.

  - The LOW’s illumination system will be used as needed for dark imaging.

- The LOW will require a beacon for navigation reference.

  - The LOW’s beacon will require a mass of 1.0 kg.

  - The LOW’s beacon will require power of 5.0 W.

  - The LOW’s beacon will be activated when needed.

- The LOW will require a drill system.

  - The LOW’s drill system will require a mass of 20.0 kg.

  - The LOW’s drill system will require power of 30.0 W.

  - The LOW’s drill system will be capable of recovering regolith from a depth of up to 2 m.

- The LOW’s drill system will require a belly cam.

  - The LOW’s belly cam will require a mass of 0.4 kg.

  - The LOW’s belly cam will require power of 3.8 W.

  - The LOW’s belly cam will be capable of imaging the drill interface with surface.

- The LOW will require a scoop.

  - The LOW’s scoop will require a mass of 0.5 kg.

  - The LOW’s scoop will be capable of recovering regolith from the surface.

- The LOW will require an XRD / XRF system.

  - The LOW’s XRD / XRF system will require a mass of 2.0 kg.

  - The LOW’s XRD / XRF system will require power of 10.0 W.

  - The LOW’s XRD / XRF system will be capable of determining the mineralogy and chemistry of the regolith.

- The LOW will require geotechnical experiments

  - The LOW’s geotechnical experiments will require a bearing plate with a mass of 1.0 kg.

  - The LOW’s geotechnical experiments will require three cone penetrometers with a total mass of 1.5 kg.

  - The LOW’s geotechnical experiments will require a shear vane of with a mass of 0.5 kg.

  - The LOW’s geotechnical experiments will be capable of determining geotechnical properties such as soil cohesiveness, density, packing, moisture, and heat.

- The LOW will require an gamma ray spectrometer.

  - The LOW’s gamma ray spectrometer will require a mass of 8.6 kg.

  - The LOW’s gamma ray spectrometer will detect the presence of thorium and uranium.

- The LOW will require two arms.

  - Each of the LOW’s arms will require a mass of 13.0 kg.

  - Each of the LOW’s arms will require power of 43.0 W.

  - One of the LOW’s arms will be capable of controlling the scoop and the geotechnical experiments and loading the SRV.

  - One of the LOW’s arms will be capable of controlling the Gamma Ray Spectrometer and the XRD / XRF.

- The LOW will require a photoemission spectrometer (PES).

  - The LOW’s PES will require a mass of 3.0 kg.

  - The LOW’s PES will be capable of measuring electrons emitted from the surface.

  - The LOW’s PES will be deployed at the first site.

- The LOW will require a photoelectric system.

  - The LOW’s photoelectric system will require a mass of 1.0 kg.

  - The LOW’s photoelectric system will be capable of determining the intensity of light.

  - The LOW’s photoelectric system will be deployed at the first site.

- The LOW will require an emission spectrometer.

  - The LOW’s emission spectrometer will require a mass of 3.0 kg.

  - The LOW’s emission spectrometer will require power of 7.0 W.

  - The LOW’s emission spectrometer will be capable of determining the chemical composition from micrometeoric impact flashes. mast will be deployed once at the initial site.

  - The LOW’s emission spectrometer will be deployed at the first site.

- The LOW will require a witness plate.

  - The LOW’s witness plate will require a mass of 1.0 kg.

  - The LOW’s witness plate will be used to observe micrometeorite impacts.

- The LOW will require a langmuir probe.

  - The LOW’s langmuir probe will require a mass of 3.0 kg.

  - The LOW’s langmuir probe will require power of 5.0 W.

  - The LOW’s langmuir probe will be capable of analyzing levitated dust.

  - The LOW’s photoelectric system will be deployed at the first site.

- The LOW will require a particle probe.

  - The LOW’s langmuir probe will require a mass of 7.0 kg.

  - The LOW’s langmuir probe will require power of 7.5 W.

  - The LOW’s langmuir probe will be capable of analyzing levitated dust.

  - The LOW’s photoelectric system will be deployed at the first site.

- The LOW will require a neutron spectrometer.

  - The LOW’s neutron spectrometer will require a mass of 1.0 kg.

  - The LOW’s neutron spectrometer will require power of 1.8 W.

  - The LOW’s neutron spectrometer will be capable of determining the H content of regolith.

  - The LOW’s photoelectric system will be deployed at the first site.

- The LOW will require a Gas Chromatograph Mass Spectrometer (GCMS).

  - The LOW’s GCMS will require a mass of 19.0 kg.

  - The LOW’s GCMS will require power of 75.0 W.

  - The LOW’s GCMS will run for four hours at each site.

  - The LOW’s GCMS will analyze the compounds and composition of regolith.

- The LOW’s GCMS will require a GCMS sample processing system.

  - The LOW’s GCMS sample processing system will require a mass of 1.0 kg.

  - The LOW’s GCMS sample processing system will prepare regolith samples with the GCMS.

- The LOW will require a tunable diode laser.

  - The LOW’s tunable diode laser will require a mass of 1.0 kg.

  - The LOW’s tunable diode laser will require power of 10.0 W.

  - The LOW’s tunable diode laser will determine the amount of water in regolith.

- The LOW will require an electron paramagnetic resonance spectroscope (EPRS).

  - The LOW’s EPRS will require a mass of 5.0 kg.

  - The LOW’s EPRS will require power of 5.0 W.

  - The LOW’s EPRS will determine the reactivity of dust.

- The LOW’s EPRS will require an EPRS sampling processing system.

  - The LOW’s EPRS sampling processing system will require a mass of 1.0 kg.

  - The LOW’s EPRS will separate regolith particles based on size (less than and greater than 100 nm).

- The LOW will require magnets.

  - The LOW’s magnets will require a mass of 0.5 kg.

  - The LOW’s magnets will determine the magnetic susceptibility of regolith.

- The LOW will require an IR bolometer.

  - The LOW’s IR bolometer will require a mass of 2.0 kg.

  - The LOW’s IR bolometer will require power of 5.0 W.

  - The LOW’s IR bolometer will determine surface temperature.

- The LOW will require UV imaging.

  - The LOW’s UV imaging will require a mass of 5.0 kg.

  - The LOW’s UV imaging will require power of 5.0 W.

  - The LOW’s UV imaging view the surface of each site.

- The LOW will require Helium-3 sensors.

  - The LOW’s He-3 sensors will determine the presence of He-3.

- The LOW will require an alpha particle (AP) spectrometer.

  - The LOW’s AP spectrometer will require a mass of 4.0 kg.

  - The LOW’s AP spectrometer will require power of 7.0 W.

  - The LOW’s AP spectrometer will detect APs emitted by radioactive gases leaking from the lunar interior.