#.# Power Subsystem

#.#.# Power Subsystem Summary

During phase 2 the power subsystem was chosen to be an RTG or ASRG powered by Plutonium-238. It was later discovered that procurement of Plutonium-238 would take 5 years and would push the launch date. The fact that only approximately 10 kg of Plutonium would be available and that the LOW using an RTG would require 20-30 kg lead us to choose a different power system. We attempted to use a type of fuel cell, but from academic advisement a total battery system was chosen. The power estimated over 1000 W was a huge undertaking for any power system of this size. A phase mission shown as Table 1 below power budget was created to reduce the total mass required to 200 W. In order not to affect the science of our mission, research was conducted to look into NASA’s current power systems and find one that could fulfill all goals set out by Team Eclipse. A total system includes solar cells, lithium ion batteries, and a power controller.

Table #1 Phase Mission Power Budget

Table 1 above shows the overall power budget separated into 6 major categories in We. In each column is a listing of estimated average load requirements for the various subsystems. Taking the totals of each load shows the total average load in each phase. The power subsystem would be designed by the maximum average load. Adding a factor of 10% 250W during the driving phase would require the most power. Table 2 below shows each subsystem’s required peek load in We-hr.

Table #2 Maximum Peek Load for subsystems

#.#.# Power Equipment

Solar power is the main power system of choice for the LOW. Similar to the ones used by NASA these solar cells will power the rover in daylight, recharge batteries, and power the electrolysis system for the fuel cell. Approximately 5m2 area of solar cells would be required to power this system. The assumptions where: 23.5 degrees for worst angle; degrade of 3.75% per year, and a mission time of 1 year.

(1)

where:

Prv = Total power required during daylight

Pe = Power required for dark

Pd = Power required for daylight

Xe = Efficiency of path to batteries

Xd = Efficiency of path from arrays to the loads

Te = Time during dark

Td = Time during light

The power calculated using the equation 1 above where as the power required during the daylight is actually the power required during the daylight plus the power required during the dark. For this power subsystem Pe is 200 We and Pd is 200 We, the time in the dark is 8 days assuming 6 days of sleep mode and 14 days of light. The total powered required was shown to be 450 We for the solar cells.

Lithium-Ion batteries are the power storage for the LOW. The 24 batteries has a total capacity of 45 kWe-hr. The batteries have multi function purpose. The batteries function as a power regulator from which all systems draw their power.

(2)

where:

Cr = Total battery capacity

Pn = Maximum peak power needed

Tn = Maximum peak time at peak power

DOD = Depth-of-discharger based on Li-Ion

Nrv = Number of batteries

nrv = Battery-to-load transmission efficiency

The battery capacity per battery for the Lithium-Ion battery is shown in equation 2. Pn*Tn is the maximum peek power required at any time for the length of time, given time from Table 2. A it was found that 256 We-hr for each of the 4 batteries. Below on figure 1 shows the battery wattage output as a variance of time.

Figure #1 Battery Capacity

#.#.# Power System Controller

PSC will be used to control the power subsystem. This PSC is separate from the main computer of the LOW for the regulating the power. The PSC will communicate with the main computer for the basic inputs. The PSC must have voltage and amperage control, and protect the LOW from the various power considerations such as overdraw, under output, power spikes, etc. The PSC will also take input from the main computer for the phase missions.

Phase mission considerations. The total output of the LOW if every system was turned on would be way over 1000 W. A power system for that kind of power would require a lot of mass. Mass that would not be feasible for this LOW. The use of Phase mission is common to NASA spacecraft. The main phases are: Landing, Initializing, Driving, Science, Communications, Sleep, and EoM & SRV. Looking at Table 1 and Table 2 the calculated power outputs are calculated into 2 kinds of load in We and peak load in We-hr.

#.#.# Power Cabling

The LOW will use standard Teflon insulated power cables able to transmit up to 20 A and 30 V. The LOW will use approximately 20 m of cable at a mass of 15 kg.

References:

Spacecraft design handbook <look up book>

Risk Analysis, Vol. 18, No. 4, 1998

A Methodology to Select a Wire Insulation for Use in

Habitable Spacecraft Todd PauloslJ and George ApostolakisZ

http://www.blackwell-synergy.com/action/showPdf?submitPDF=Full+Text+PDF+%281%2C286+KB%29&doi=10.1111%2Fj.1539-6924.1998.tb00362.x&cookieSet=1