#.# Power Subsystem

#.#.# Power Subsystem Summary

During phase 2 the power subsystem was chosen to be an RTG or ASRG powered by Plutonium-238. It was later discovered that procurement of Plutonium-238 would take 5 years and would push the launch date. The fact that only approximately 10 kg of Plutonium would be available and that the LOW using an RTG would require 20-30 kg lead us to choose a different power system. The power estimated over 1000 W was a huge undertaking for any power system of this size. A phase mission power budget was created to reduce the total mass required to 250 W. In order not to affect the science of our mission, research was conducted to look into NASA’s current power systems and find one that could fulfill all goals set out by Team Eclipse. A multi-power system was chosen from existing technology which includes fuel cells, solar cells, Lithium-Ion batteries, power controller, and electrolysis equipment.

Table #1 Phase Mission Power Budget

Table 1 above shows the overall power budget separated into 6 major categories in We. In each column is a listing of estimated average load requirements for the various subsystems. Taking the totals of each load shows the total average load in each phase. The power subsystem would be designed by the maximum average load. Adding a factor of 10% 250W during the driving phase would require the most power. Table 2 below shows each subsystem’s required peek load in We-hr.

Table #2 Maximum Peek Load for subsystems

#.#.# Power Equipment

Solar power is the main power system of choice for the LOW. Similar to the ones used by NASA these solar cells will power the rover in daylight, recharge batteries, and power the electrolysis system for the fuel cell. Approximately 5m2 area of solar cells would be required to power this system. The assumptions where: 23.5 degrees for worst angle; degrade of 3.75% per year, and a mission time of 1 year.

(1)

where:

Prv = Total power required during daylight

Pe = Power required for dark

Pd = Power required for daylight

Xe = Efficiency of path to batteries

Xd = Efficiency of path from arrays to the loads

Te = Time during dark

Td = Time during light

The power calculated using the equation 1 above where as the power required during the daylight is actually the power required during the daylight plus the power required during the dark. For this power subsystem Pe is 250 We and Pd is 500 We, the time in the dark is 10 days assuming 4 days of sleep mode and 14 days of light. The total powered required was shown to be 1028 We for the solar cells.

Fuel cells are the secondary power subsystem for the LOW. The <Fuel cell name> is the fuel cell from the Space Shuttle. It will be the main power source while the LOW is not using the solar cells. During the dark the LOW will have 10 days of full power and 4 days of degraded mode. Assuming that there is 14 days of light and 14 days of dark, the LOW will be capable for long term scientific exploration. 1 single fuel cell will power the LOW. 4 total fuel cells are chosen because each last 2400 hours of use and some redundancy for when one fails. 300 days total of dark capability. When one has received 75% of its useful life the next time the next fuel cell will be turned on.

Water storage for the fuel cell was carefully considered. 2 spherical titanium tanks will house the H2O in a safe pressure & temperature environment. Each tank will have lead lines and a pump from the fuel cells. 4 spherical titanium tanks will house the H2 and O2 gases. 8 many compressors and connectors from each of the gas tanks will connect to the fuel cells. A switcher comprised of a compressor/pump will be place at the leading connection to the 4 fuel cells. The multiple tanks are chosen because of easier pressure control, volume placement, and extra redundant capability.

Lithium-Ion batteries are the power storage for the LOW. The 4 batteries has a total capacity of 1000 We-hr. The batteries have multi function purpose. One the batteries function as a buffer from the wild output of the fuel cells and act as a power regulator from which all systems draw their power. Two store power for peek power requirements during the phase missions.

(2)

where:

Cr = Total battery capacity

Pn = Maximum peak power needed

Tn = Maximum peak time at peak power

DOD = Depth-of-discharger based on Li-Ion

Nrv = Number of batteries

nrv = Battery-to-load transmission efficiency

The battery capacity per battery for the Lithium-Ion battery is shown in equation 2. Pn*Tn is the maximum peek power required at any time for the length of time, given time from Table 2. A it was found that 256 We-hr for each of the 4 batteries. Below on figure 1 shows the battery wattage output as a variance of time.

Figure #1 Battery Capacity

Electrolysis equipment will have to be developed for the LOW mission. This equipment is the only device on the LOW that will not have tech level readiness of 9. The technology exists on earth but not for spacecraft. The development of such equipment should take no less then 2 years and be ready for the LOW by then. The electrolysis equipment will use 250 We and 4 extra days to divide all the water into its primary components H2 and O. During the entire daylight time the equipment will fill up the H2 and O2 tanks for use in the dark times. Because of this it is recommended that most dark missions take place during the dark time. Even with this equipment running it will not prevent the LOW from doing scientific experiments during the daytime.

#.#.# Power System Controller

PSC will be used to control the power subsystem. This PSC is separate from the main computer of the LOW for the regulating the power. The PSC will communicate with the main computer for the basic inputs. The PSC must have voltage and amperage control, and protect the LOW from the various power considerations such as overdraw, under output, power spikes, etc. The PSC will also take input from the main computer for the phase missions.

Phase mission considerations. The total output of the LOW if every system was turned on would be way over 1000 W. A power system for that kind of power would require a lot of mass. Mass that would not be feasible for this LOW. The use of Phase mission is common to NASA spacecraft. The main phases are: Landing, Initializing, Driving, Science, Communications, and EoM & SRV. Looking at Table 1 and Table 2 the calculated power outputs are calculated into 2 kinds of load in We and peak load in We-hr.

#.#.# Evaluation of the System

The combination of the solar cells and fuel cells is a smart direction from using nuclear power. It cannot be helped that the H2O tanks be large as H2O is incompressible for the most part. Since the technology for miniature cryogenics isn’t around the LOW must keep H2 and O2 in gaseous form. The biggest problem is the density of those gases. Future technologies may allow a smaller system by allowing cryogenic storage of H2 and O2 in liquid compressed form. The use of this technology may allow fuel from the moon as H2O to be developed on the moon never having to be brought there. The exploration of this system can allow further development for manned space missions to the moon.

References:

Fuel Cell Power Plants. Government article no stated author <http://spaceflight.nasa.gov/shuttle/reference/shutref/orbiter/eps/pwrplants.html>

Spacecraft design handbook <look up stuff>